# DS1307 Real-Time Clock Commands

The DS1307.h include file provides both high- and low-level support for the DS1307 real-time clock chip, as well as inexpensive breakout boards like the TinyRTC which come packaged with a backup battery and additional eeprom. Insert the following directive in your code to make these new commands available:

#include <DS1307.h>

Here follows a list of the commands..

DS1307_Enable(flag)

- enables the clock when flag is TRUE,

- disables the clock when flag is FALSE

DS1307_ResetClock

- resets clock completely to manufacturer’s original condition,

- time to 00:00:00, day of the week to 01, date to 01/01/00,

- also sets 24-hour mode and enables the clock.

DS1307_SetTime(hour, minute, second)

- sets the time only: hours, minutes, seconds,.

- also sets 24-hour mode and enables the clock.

DS1307_SetDate(DOW, date, month, year)

- sets the date only: day of week (1-7), date, month, year,

- there is no error detection for out-of-range dates, (e.g., April 31)

DS1307_SetClock(hour, minute, second, DOW, date, month, year)

- sets the entire clock: hours, minutes, seconds, day of week, date, month, year.

- there is no error detection for out-of-range dates, (e.g., April 31)

- also sets 24-hour mode and enables the clock.

DS1307_ReadTime(hour, minute, second, flag)

- reads the time only: hours, minutes, seconds, a.m. or p.m.,

- flag = FALSE means a.m.,

- flag = TRUE means p.m.

DS1307_ReadDate(DOW, date, month, year)

- reads the date only: day of week (1-7), date, month, year

DS1307_ReadClock(hour, minute, second, flag, DOW, date, month, year)

- reads the entire clock: hours, minutes, seconds, flag, day of week, date, month, year

- flag = FALSE means a.m.,

- flag = TRUE means p.m.

DS1307_SetHourMode(12|24)

- sets the hour mode,

- 12 = 12-hour

- 24 = 24-hour

- any other value defaults to 24-hour mode

DS1307_ReadHourMode(12|24)

- returns the current hour mode within clock

- 12 = 12-hour

- 24 = 24-hour

DS1307_SetSQW(rate)

- sets the square wave output pin mode:

- 0 = disable square wave output

- 1 = 1 Hz output

- 4 = 4096 Hz

- 8 = 8192 Hz

- 32 = 32768 Hz

- any other value defaults to 1 Hz

DS1307_Write(address, value)

- writes to the internal registers or RAM,

- registers: 0x00 to 0x07

- RAM: 0x08 to 0x3F

- writing beyond this wraps around to the register space again, so be careful with multibyte writes

DS1307_Read(address, value)

- reads from the internal registers or RAM,

- see the notes, above.