## Great Cow BASIC Command Reference

Preliminary version

(Not suitably comprehensive for end users.)

Last updated: 9/7/2006

Legend to highlighting:

Tested and working

Tested and not working

Not tested

Not implemented

Commands:

| Command | Explanation |
| --- | --- |
| # Flow Control |  |
| IF condition THEN … END IF | Execute commands between IF and END IF, if condition is true. |
| IF condition THEN command | Short form of IF. Does not require END IF. Command must be on same line as IF. |
| GOTO label | Jump to label |
| label: | Defines a label |
| GOSUB label | Call label as a subroutine |
| RETURN | Return from a GOSUB |
| SubName [(vars)][#NR] | Call up sub SubName, and pass vars to it. If vars are specified, then brackets must be included. #NR stands for No Return, and will stop the sub from returning values of integer variables. |
| SUB Subname [(vars)] | Used to mark the start of a subroutine |
| EXIT SUB | Exit from a subroutine (Only found in subs) (interchangeable with EXIT FUNCTION) |
| END SUB | Marks the end of a subroutine (interchangeable with END FUNCTION) |
| EXIT FUNCTION | Exit from a function (Only found in functions) (interchangeable with EXIT SUB) |
| FUNCTION functionname [(vars)] … END FUNCTION | Same as for sub, except a value can be returned automatically ie: Var = FunctionName (var2) (Note: When defining a function, be sure to return a value. To do this, set FunctionName to some value/var.) |
| FOR var = min TO max STEP step …. NEXT | Loop (max-min) times. Var will start at min and be incremented every loop until it is equal to max. During this time, all code up to NEXT will run. Step is number to add to var each loop. Step can be a negative. |
| DO WHILE¦UNTIL condition … LOOP | Repeat code while/until condition is true |
| CALL sub EVERY ticks | Call up sub every ticks of the interrupt. Set ticks to 0 to stop call for a particular sub. (Interrupt must be defined) |
| # Variable |  |
| [LET] var1=var2 | Set var1 to var2. var2 can be given as a function. LET is optional, and exists solely for people familiar with other BASIC dialects. |
| ROTATE var {LEFT¦RIGHT} | Rotate var 1 bit left/right |
| SET var.bit, 0¦1 | Set bit of var to 0 or 1 (OFF or ON accepted) |
| DIM ArrayName(size) | Initialize an array. |
| DIM VarName AS type | Declare a non-byte variable. Only “word” is a valid type at present. |
| Compiler (Commands in main file will override those in includes, and commands in includes will override those in nested includes) |  |
| # #include “file.h” | Include the subs in file.h into the program Note: using “” will read the file from the same directory as the program. If <> are used, the file will be read from the default include folder. |
| # #define CONST value #define CONST =sum | Create a constant called CONST, and set it to value, or result of sum. |
| # #chip MODEL, MHz | Specifies model and speed of PIC |
| # #osc Type | Type is HS/LP/etc. |
| # #config settinglist | Settinglist is list of config values in .inc file (WDT_OFF, etc.) |
| # #IFDEF constant value[,value2,etc.] … #ENDIF | If constant != value/value2/etc, then the code between value can also be another constant #IFDEF and #ENDIF will be removed from the compiled code. |
| # #SCRIPT … #ENDSCRIPT | Used to make a script which GCBASIC runs. Use for creating code that can be run on different PIC chips. See the script commands section for more info |
| # #int frequency | Set up an interrupt of the given frequency. Used in conjunction with the CALL EVERY command. Frequency is Hz (app. range on 20 MHz: 76-19,000). |
| # #startup subname | Used in include files to define a subroutine that must be called if anything from that file is used. |
| # #RAM ramsize | Specifies the amount of RAM present on the PIC chip. Not compulsory, but should be used if arrays are in use. |
| Important: Compiler directives should not be in subs or functions! (Except #IFDEF, #ENDIF, #SCRIPT, and #ENDSCRIPT) |  |
| # RS 232 Commands (lowlevel\rs232.h) |  |
| # InitSer (channel, rate, start, data, stop, parity, invert) | Channel = 1,2,3 Rate = r300, r600, r1200, r2400, r4800, r9600, r19200 Start = start bits (usually 1), add +WaitForStart to make the program wait for the start bit when receiving. Data = data bits (usually 8) Stop = stop bits (usually 1) Parity = none, odd, even Invert = normal, invert |
| # SerSend (channel, data) | Data is byte to send using channel |
| # SerReceive (channel, variable) | Received byte from channel will be written to variable |
| # SerPrint (channel, String$) | Send String$ over given channel |
| A/D Commands (lowlevel\a-d.h) |  |
| # ReadAD (port) | Initializes, reads the analog value of port, and restores ports to digital. Port is AN0-ANx, depending on PIC version. Incorporates several of the commands below. |
| # ConvertAD | Starts A/D conversion, and waits for it to finish |
| # ADOn | Turns on A/D, and sets all ports to analog |
| # ADPort (port) | Selects the port to read |
| # ADFormat (Format_Left ¦ Format_Right) | Left ignores highest bits in ADRESH. Right ignores lowest in ADRESL. |
| # ADOff | Disables A/D, and sets all ports to digital |
| EEPROM Commands (lowlevel\eeprom.h) |  |
| # EPRead (address, var) | Read value from EEPROM. Address is location, var is var to place data in. |
| # EPWrite (address, data) | Write value to EEPROM. Address gives location, data is byte to write. |
| # ProgramWrite (Address H, Address L, Data H, Data L) | Write the 14-bit value given by Data to the program memory location given by Address. (Only supported by some chips) |
| # ProgramRead (Address H, Address L, Data H, Data L) | Read the 14-bit value given by Data from the program memory location given by Address. (Only supported by some chips) |
| # ProgramErase (Address H, Address L) | Erase the 32 bytes of program memory starting at the given address. This must be done before writing to a location. For more details, please see the relevant manual for your PIC chip. |
| PWM Commands (lowlevel\pwm.h) |  |
| # InitPWM | Sets up PWM. PWM frequency in KHz is given by the PWM_Freq constant, which defaults to 38 (KHz). PWM Duty cycle is set using the PWM_Duty constant (Default 25 %) |
| # PWMOn | Enables PWM by switching the CCP module to PWM mode |
| # PWMOff | Disables PWM by switching the CCP module off. |
| SRF04 Distance Sensor Commands (srf04.h) |  |
| # USDistance (port no.) | Function to read a distance from an SRF-04 distance sensor. The sensor must have been defined correctly – see the include file for more information. |
| Sound Commands (lowlevel\sound.h) |  |
| # Tone (Frequency, Duration) | Frequency is Hz/10; Duration is in 10 ms units. |
| # ShortTone (Frequency, Duration) | Frequency is Hz/10, Duration is in 1 ms units |
| LCD Display Commands (lowlevel\lcd.h) |  |
| # PRINT String$ | Write the string to the LCD, starting at the current cursor location. |
| # LOCATE column, line | Set the cursor to the given position |
| # PUT (location, ASCII char) | Write the specified ASCII character to the given location |
| # x = GET (location) | Return the ASCII character at the given position |
| # CLS | Clear the LCD |
| # LCDInt (Value) | Write the given value to the LCD, at the current cursor location. |
| # LCDHex (Value) | Write the given value to the LCD, at the current cursor location. The value will be shown in hexadecimal format. |
| 7-Segment Display Commands (lowlevel\7segment.h) |  |
| # DisplayValue (Display Select, Value) | Display Select is 1, 2, 3 or 4, corresponding to the display that is to be output to. Value is the value between 0 and 9 to be shown on the screen. |
| # DisplayChar (Display Select, ASCII Code) | Display Select is 1, 2, 3 or 4, corresponding to the display that is to be output to. ASCII Code is the character code to be shown. Note: This can be given as a single character, enclosed in quotes) |
| # String handling Functions |  |
| # Var = LEN(String$) | Returns the length of the specified string. |
| # OutString$ = MID$(InString$, position, length) | Returns the given portion of InString |
| # Misc. Commands/Functions |  |
| # WAIT length units | Length 0-255, units = us, 10us, ms, 10ms, s, m, hour |
| # WAIT WHILE¦UNTIL condition | Wait until condition is true |
| # SLEEP time | Equivalent to Wait time s. Included for compatibility with other BASIC dialects |
| # DIR PORTBIT IN¦OUT | PORTBIT is A0, A1, etc. IN/OUT are obvious. On 10/12 series chips, use IO0, IO1, etc. |
| # DIR PORT DirByte | PORT is A, B, etc. DirByte is a byte specifying the value for the applicable TRIS register (Can be a var or constant) On 10/12 series chips, PORT is omitted |
| # PULSEOUT pin, time units | Set pin high for time. |
| # POT pin, output var | Measure the period of an R/C oscillator connected to pin, then write result to output var. This command should be used sparingly, as each use results in 20 assembly instructions. If the same pin is to be read twice, use a function with this command in it. |
| # ‘ ; REM | Used as comments. |
| # Var = PEEK (high location, low location) | Set Var to the contents of the given memory location |
| # POKE (high location, low location, value) | Set the given RAM location to the given value |
| # Var = Random | Returns a pseudo-random integer between 0 and 255 |
| # Swap (Var1, Var2) | Swaps the values of Var1 and Var2 |
| # Swap4(Var) | Swaps the nibbles in Var |

# Compiler script commands:

| Command | Explanation |
| --- | --- |
| IF condition THEN … END IF | Execute the script within if condition is true. Note that constants can be used as vars, and vars must not be used. Otherwise, this is similar to the normal IF. (Short form is invalid – END IF must be present) |
| ERROR message | Add message to the error listing |
| define = sum | Set the compiler constant to the value of sum |
| Note: These commands are executed by GCBASIC, and are useful for setting constants to a particular value inside include files. They are not downloaded to the PIC. Constants are treated as variables are normally treated –they can be set. Constants must have been defined before they are used in an IF, either by setting them, or using #define. |  |

# Functions:

| # Function | # Explanation |
| --- | --- |
| x * y, x / y, x + y, x - y | Self explanatory |
| x AND y | Logical AND of x and y. Spaces must be included |
| x OR y | Logical OR of x and y. Spaces must be included |
| x XOR y | Logical XOR of x and y. Spaces must be included |
| NOT x | Inverts x. Space must be included |
| x & y, x ¦ y, x # y, ! x | Short form of AND, OR, XOR, and NOT respectively. Spaces are optional with the short form. |
| x = y | Returns 0 if not equal, 255 if equal |
| x <> y | Returns 255 if not equal, 0 if equal |
| x > y | Returns 255 if x is more than y, 0 otherwise |
| x < y | Returns 255 if x is less than y, 0 otherwise |
| x >= y | Returns 255 if x is equal to or more than y, otherwise 0. |
| x <= y | Returns 255 if x is less than or equal to y, otherwise 0. |

Functions must have “var =” before them (minus quotes), where var is the name of the variable that is to be set.

Order of operations:

- ()

- +, -

- AND, OR, XOR, NOT

# Conditions

- = (equal)

- <> or ~ (not equal)

- < (less than)

- > (greater than)

- <= (less than or equal)

- >= (greater than or equal)

Conditions are basically sums, with either zero (false) or non-zero (true) results.

# Built-in #defines (used for #IFDEF)

# ChipMHz

- ChipName

- OSC

Built-in #defines (used for IFs, WAITs, etc.)

| Name | Value |
| --- | --- |
| ON | 1 |
| OFF | 0 |
| TRUE | 255 |
| FALSE | 0 |

NB. Other #defines are set by the built-in include files.

Time Units

| Units | GCBASIC units |
| --- | --- |
| Microseconds | us |
| Microseconds * 10 | 10us |
| Milliseconds | ms |
| Milliseconds * 10 | 10ms |
| Seconds | s |
| Minutes | m |
| Hours | h |

Data Types:

| Type | Size | Range |
| --- | --- | --- |
| Byte | 8 bits | 0 to 255 |
| Word | 16 bits | 0 to 65535 |
| Float | 24 bits | ? |

Known Limitations:

- Chip Speed: 0.1-60 Mhz

- Lines of GCBASIC code (including subs): 4000

- Assembly Program: 4000 commands

- Include files: 20 (includes built-in include files)

- Constants: 200

- Variables: approx. 380 or available RAM on PIC (whichever is less)

- Subs (including in include files): 200

- Array size: 80 bytes (Avoid large arrays wherever possible)

- String length: 20 characters default, 80 if declared using DIM

- Strings: 100

Things which MUST be defined in EVERY program:

#chip model, MHz

One of #config setting1, setting2, etc. OR #osc source

(These can be defined in an include file instead. If something is defined in an include file and the main program, the definition in the main program will be used.)