# MHS-3225A Dual-channel DDS Signal Generator 25MHz Sine wave 200MSa/s DDS signal source frequency meter TTL output

MHS-3200A DDS signal generator is Minghe Electronics launched in 2016 a cost-effective multi-purpose signal generator product, instrument uses large scale integrated circuits and high-speed FPGA MCU microprocessor, the internal circuit to take surface mount technology has greatly enhanced the instrument\'s noise immunity and service life. Display interface using LC1602 LCD display is divided into two lines, the top line shows the current frequency, the following line displays additional parameters or function variable. This instrument signal generation, waveform scanning, as well as the use of parameter measurement has great advantages, is an electronic engineer, electronic laboratories, production lines and teaching, research and the ideal test, measurement equipment.

It is worth mentioning that we have two channels are completely independent, the maximum frequency can reach 25MHZ.

MHS-3200A instrument model of the last two digits 00 indicate limits (MHz) frequency sine wave instrument on the model.

Example: MHS-3200A (6MHz), 6MHz means the model sine wave output frequency up to 6MHz

1, direct digital synthesis (DDS) technology, FPGA design, low power consumption;

2, dual output, can work in sync phase adjustable;

3, with up to 600 seconds of linear and logarithmic sweep sweep function;

4, with a sine wave, the basic function waveform triangle wave, square wave, sawtooth rise, falling sawtooth and variable duty cycle of the pulse wave,

5, with M0 ~ M9 total of 10 sets of parameters stored bits M0 boot automatically transferred out of data;

6, at 15MHz or less, the sharpest of up to 15Vp-p, more than 12MHz, the biggest reach 8Vp-p;

7, built sophisticated -20dB attenuator reach the minimum amplitude resolution 1mV;

8, with 120% ~ + 120% DC bias function;

9, pulse duty cycle adjust accurate to 0.1%;

10, with four variable phase difference of TTL output;

11, has a frequency measurement, period measurement, positive and negative pulse width measurement, duty cycle measurements and counting function;

12, four optional frequency measurement gate time, which strike a balance between speed and accuracy;

13, all parametric EQ calibration can be performed by internal procedures;

14, powerful communications features, completely open communications protocol, the secondary development becomes very simple;

15, after the connection with the PC, the PC can be used to control the instrument,

Dual simultaneous output 15KHz, peak value 5Vp-p sine

wave

Dual simultaneous output 15KHz, peak to peak square wave 5Vp-p

Dual simultaneous output 15KHz, peak value of the triangular wave 5Vp-p

CH2 channel output 15KHz peak value 15Vp-p sine wave

CH2 channel output 15KHz peak value 0.015Vp-p (ie 15mv) sine wave

CH2 channel output 15KHz peak value 0.016Vp-p (ie 16mv) sine wave, step value can be done 1mv, quite accurate!

CH2 channel output 15KHz peak value 0.018Vp-p (ie 18mv) sine wave